# Comprehensive Life Cycle Assessment and Phytonutrient Optimization in Controlled Environment Microgreen Agriculture

## Introduction to the Optimization of High-Density Organic Yields

The cultivation of microgreens and sprouts within highly customized Controlled Environment Agriculture (CEA) systems represents a paradigm shift in urban and precision agricultural production. These systems isolate the growing environment, allowing for the absolute control of lighting, nutrient delivery, and substrate conditions. This control is leveraged to maximize the output of specific phytonutrients, such as glucosinolates and antioxidants, which are present in species like broccoli, sunflower, and fenugreek at concentrations vastly exceeding those found in their mature counterparts1. However, the intensity of this environmental control inherently requires substantial inputs of electricity, synthetic hardware, and processed biological media.

To ensure that the operation is both environmentally and economically sustainable, a rigorous Life Cycle Assessment (LCA) is required to quantify the environmental impact of the product from raw material extraction to end-of-life disposal. The development of advanced farm management software, specifically the Cotyledon application, designed to orchestrate these complex biological and operational parameters, provides an unprecedented opportunity to move away from static environmental audits. Instead, by integrating real-time operational data with established life cycle inventory databases, agricultural operations can dynamically calculate the environmental footprint of every single harvest. The specific architectural footprint of this operation relies on a dense vertical integration strategy utilizing stainless steel wire racks configured as six-shelf, 2x4 ft units, with five active grow shelves per rack. Each shelf is illuminated by multiple 4-ft LED grow bulbs and houses standard 10 in x 20 in flats.

The ensuing analysis meticulously evaluates the LCA data associated with indoor LED lighting across various manufacturers (Barrina, Mars Hydro, VivoSun), organic hand-watered hydroponic nutrients, the Cotyledon default hemp mat substrate alongside comparative analyses of historical alternatives like soils and silicone meshes, and the upstream supply chain of organic seeds sourced from True Leaf Market. This comprehensive review establishes the foundational dataset necessary for a software-driven, ecologically optimized microgreen production facility.

## Framework for Agricultural Life Cycle Assessment and Database Architecture

Life Cycle Assessment is a standardized, science-based analytical methodology used to evaluate the environmental burdens associated with a product, process, or service throughout its entire life cycle2. In accordance with the International Organization for Standardization guidelines, specifically ISO 14040 and ISO 14044, a complete LCA is structured around four iterative phases: the definition of the assessment's objective and boundaries, the compilation of an inventory of inputs and outputs (Life Cycle Inventory or LCI), the assessment of potential environmental impacts (Life Cycle Impact Assessment or LCIA), and the interpretation of the results2.

For a highly customized indoor microgreen operation, the functional unit—the reference to which all inputs and outputs are mathematically normalized—is typically defined as one kilogram (1 kg) of fresh weight harvested microgreens5. The system boundary for this assessment follows a "cradle-to-grave" model. This encompasses the extraction and manufacturing of structural raw materials (such as stainless steel and silicone), the upstream agricultural production of organic seeds, the continuous operational inputs (electricity, water, organic fertilizers), and the end-of-life disposal or recycling of biological waste and infrastructural components2.

To quantify these impacts, the assessment relies on specific LCIA midpoint indicators. The most critical indicators for a CEA facility include Global Warming Potential (GWP), Eutrophication Potential (EP), and Acidification Potential (AP). Global Warming Potential, measured in kilograms of carbon dioxide equivalents (), quantifies the radiative forcing impact of greenhouse gases like , , and  over a standard 100-year time horizon4. Eutrophication Potential, measured in phosphate equivalents () or nitrogen equivalents (), tracks the potential for nutrient pollution in aquatic and terrestrial ecosystems, primarily driven by nitrogen and phosphorus emissions9. Acidification Potential, measured in sulfur dioxide equivalents (), evaluates the release of protons into ecosystems, largely from agricultural ammonia and fossil fuel combustion4.

The underlying data required to construct this assessment within the Cotyledon software must be sourced from highly verified, peer-reviewed databases. Ecoinvent serves as the most extensive and versatile background database, providing robust lifecycle inventories for electricity grids, plastic and metal manufacturing, and transport logistics3. For specific agricultural inputs, the French database Agribalyse offers highly disaggregated data on food and agricultural systems13. Furthermore, the USDA's Federal LCA Commons provides critical baseline data for North American agriculture, offering precise unit process data for field crop energy consumption, seed production carbon footprints, and regional electricity emission factors across the United States16. Integrating these diverse databases allows the software to track the geographically dispersed origins of organic seeds and materials.

## Structural Hardware: Embodied Carbon and Lifespan Amortization

The infrastructural backbone of the microgreen system consists of heavy-duty stainless steel wire shelving. The specific configuration utilizes 2x4 ft racks with six shelves, dedicating five shelves to active microgreen propagation. Within these shelves, standard 10 in x 20 in flats are deployed. Historically, to provide structural support for the microgreen roots in a purely liquid hydroponic environment without a fibrous medium, the system employed silicone and stainless steel mesh screens laid directly inside the flats. While Cotyledon now defaults to hemp mats, these reusable meshes are actively maintained in the LCA database to compare their sustainability impact and resource footprint against biological alternatives.

The integration of refined metals and synthetic polymers into an agricultural system shifts a significant portion of the environmental burden from the biosphere (agricultural land use and soil degradation) to the technosphere (industrial manufacturing and mineral extraction). Environmental Product Declarations (EPDs) for steel components reveal a substantial initial carbon footprint. The manufacturing phase (modules A1-A3 in standard LCA nomenclature) for one kilogram of galvanized, powder-coated, or stainless steel yields a Global Warming Potential ranging between 2.78 and 3.62 19. This footprint encompasses the extraction of iron ore, the intense thermal energy required for smelting, and the subsequent forming, weaving, and protective coating processes required to produce steel wire racks and fine mesh screens.

Similarly, silicone—a synthetic polymer composed of alternating silicon and oxygen atoms (siloxanes)—requires the highly energy-intensive carbothermic reduction of silicon dioxide into metallurgical-grade silicon, followed by complex chemical syntheses. While silicone is highly durable, thermally stable, and chemically inert, making it an ideal, food-safe root support structure, its initial embodied energy is exceptionally high.

However, the raw manufacturing footprint provides an incomplete and potentially misleading picture when assessing the true sustainability of indoor agriculture. The critical metric in structural LCA is lifespan amortization. Stainless steel shelving and high-grade stainless or silicone root meshes possess exceptional durability, often carrying a reference service life of up to 50 years under standard indoor conditions19. When the initial heavy burden of  per kilogram of steel is divided across decades of continuous, 14-day microgreen growth cycles, the per-functional-unit impact (per kg of harvested microgreens) drops to fractions of a gram.

Research modeling the life cycle of building-integrated microgreen systems demonstrates that durable infrastructure, despite its high upfront cost in carbon, accounts for only about 11.3% (approximately 2.0 ) of the total systemic emissions over its operational life22. The structural stability of reusable stainless and silicone meshes bypasses the recurring life-cycle costs, transportation emissions, and continuous resource depletion associated with single-use peat or synthetic fibrous pads, demonstrating that high-embodied-carbon materials are environmentally superior when their longevity is maximized and their end-of-life recyclability (LCA module D) is effectively managed7.

## Substrate Profiles: Organic Fibers vs. Liquid Matrices

The default cultivation model for Cotyledon utilizes organic hemp mats laid inside the 10x20 inch flats, saturated with hydroponic nutritional irrigation23. However, to ensure a rigorous comparative life-cycle assessment, the software continuously evaluates the sustainability impact and resource footprint of this default against historical and alternative substrates, including mixed soils, coco coir, and the previously utilized silicone or stainless steel meshes.

### Peat and Mixed Soils

Peat moss has historically been a foundational element in mixed soil mediums due to its excellent water retention, low bulk density, and structural stability. However, from a rigorous life-cycle perspective, peat is profoundly unsustainable. Peat bogs are vital, slow-forming terrestrial carbon sinks. The extraction of peat actively degrades these delicate ecosystems and releases immense volumes of sequestered carbon dioxide and methane into the atmosphere25. In comparative LCAs of horticultural growing media, peat-based substrates consistently exhibit the highest global warming potential and terrestrial ecotoxicity indicators among all biological media25. Consequently, strictly organic operations aiming for a minimized ecological footprint must actively phase out peat in favor of renewable alternatives.

### Coco Coir

Coco coir is a byproduct of the coconut industry, repurposed as a highly effective, renewable horticultural substrate23. Despite its biological origin and high lignin content that resists rapid decomposition, the LCA of coco coir reveals significant environmental hotspots. The extraction of coir pith requires a retting process—soaking the coconut husks in brackish water for extended periods to separate the fibers. This anaerobic fermentation process results in the direct emission of biogenic greenhouse gases, specifically estimated at 0.0328 kg  and 0.0457 kg  per functional unit of processed coir7. Furthermore, coir pith has a demonstrably high impact on freshwater consumption (0.00066  per functional unit) and necessitates intercontinental shipping from tropical agricultural zones (e.g., India, Sri Lanka) to North America, drastically inflating its transportation (Scope 3) carbon footprint. Agronomically, while loose coir provides excellent root aeration, it often proves excessively messy during the precise harvesting required for microgreens, contaminating the final product28.

### The Chosen Default: Hemp Mats

Bast fibers, specifically the hemp mats chosen as the Cotyledon default, offer highly competitive environmental and agronomic advantages for microgreen production. Hemp is a vigorous, fast-growing crop that establishes a dense canopy rapidly, suppressing weeds and requiring minimal synthetic pesticides or herbicides29. When processed into non-woven grow mats, hemp provides an ideal capillary structure that balances water retention with vital root zone aeration31. Jute similarly offers high tensile strength and excellent moisture retention, providing a stable anchoring matrix for dense sprout root systems32.

The life cycle of these mats involves agricultural cultivation, mechanical decortication (separating the bast fiber from the inner woody core), and physical manufacturing processes like needle-punching or hydro-entangling to form the mats. Because these processes are largely mechanical rather than complex thermal or chemical reactions, their embodied energy remains remarkably low compared to synthetic alternatives30. Furthermore, during their rapid growth phase, hemp plants actively sequester atmospheric carbon.

While LCA methodologies continuously debate the permanent accounting of biogenic carbon storage for short-lived agricultural products, the organic end-of-life scenario—composting the root-bound mats—ensures that the organic matter is returned to the soil, functioning in a circular ecological loop35. Agronomic studies confirm the efficacy of these materials; hemp mats have been shown to yield superior fresh and dry shoot weights in microgreens compared to several other hydroponic pads, optimizing the output side of the LCA equation by maximizing the harvestable mass per unit of input energy24.

## LED Grow Lighting: Energy Footprints and Phytochemical Elicitation

In indoor vertical farming, the use phase of electrical equipment—specifically LED grow lighting and climate control HVAC—overwhelmingly dominates the total energy consumption and the resultant Global Warming Potential of the facility5. A standard building-integrated microgreen facility can attribute up to 54% of its total carbon emissions directly to electricity usage, with LED lighting alone accounting for roughly 4.4  per kg of harvested crop22.

The LCA of an LED luminaire is distinctly bifurcated between its manufacturing phase (wafer fabrication, aluminum extrusion for heat sinks, printed circuit boards) and its operational lifespan. While the fabrication of advanced LED diodes involves rare earth metals and energy-intensive cleanrooms, the operational phase accounts for the vast majority of the environmental impact over a standard 50,000-hour lifespan4. Therefore, selecting fixtures with the highest Photosynthetic Photon Efficacy (PPE)—measured in micromoles of photosynthetically active photons per joule of electrical energy ()—is the most effective strategy for reducing the operation's carbon footprint.

### Hardware Profiles: Barrina, Mars Hydro, and VivoSun

The system under analysis utilizes two to three 4-ft LED grow bulbs per shelf on 2x4 ft racks. The hardware landscape includes several prominent manufacturers, each offering different spectral and energy profiles suitable for integration into the Cotyledon software's database.

Barrina: The Barrina T8 4ft LED strips (often packaged in 6-packs) represent a highly modular and accessible approach to vertical farming. A 6-pack draws approximately 180W to 252W (30W to 42W per individual tube) depending on the specific model generation40. With up to 1152 individual LEDs in a 252W array, they provide a full-spectrum output (often tailored as pinkish white or yellow 5000K)40. The 100% aluminum housing ensures adequate passive thermal dissipation, extending diode life without the need for mechanical cooling fans41. Placing two to three bulbs per 2x4 ft shelf draws between 60W and 126W for an 8 sq ft area (7.5 to 15.7 W/sq ft). This aligns perfectly with the lower-end vegetative growth requirements suitable for early-stage microgreens, prioritizing energy efficiency over extreme intensity43.

Mars Hydro and VivoSun: These brands typically produce high-output planar quantum board or bar-style fixtures targeting dense canopies and high-light flowering crops, but their dimmable bar fixtures are highly adaptable for vertical racks. VivoSun offers the AeroLight and LumaLight series, while Mars Hydro is recognized for the FC series. Wattages range from 100W micro-fixtures up to 800W arrays designed for large 4x4 ft or 5x5 ft footprints43. These modern fixtures leverage high-bin Samsung diodes (such as the LM301B or LM301H EVO) capable of exceptional efficacies ranging from 2.7 to 2.96 44. For a 2x4 ft shelf, a dimmed 200W fixture from either brand would provide incredibly uniform coverage and deep canopy penetration, though it represents a higher capital expenditure and embodied carbon footprint during the manufacturing phase compared to simple T8 strips44.

### Spectral Tuning and Phytonutrient Trade-Offs

The core objective of the Cotyledon software is not merely raw biomass accumulation, but the maximization of specific phytonutrients and known nutritional benefits. Light quality (spectrum), quantity (Photosynthetic Photon Flux Density - PPFD), and photoperiod are the primary abiotic elicitors of secondary plant metabolites22.

Broccoli microgreens are highly prized for their exceptional concentration of glucoraphanin, the organic precursor to sulforaphane. Sulforaphane is a potent activator of the Nrf2 transcription factor, which regulates cellular antioxidant defense and detoxification pathways in the human body48. Rigorous analytical research indicates that broccoli sprouts and microgreens contain 10 to 100 times more glucoraphanin by weight than mature broccoli florets, making them functional superfoods2.

The biosynthesis of these phytochemicals is directly modulated and upregulated by specific light wavelengths:

- Blue Light (400-500 nm): Elevated proportions of blue light actively stimulate the production of glucosinolates, polyphenols, and essential mineral elements22. It acts as a mild abiotic stressor, increasing the overall phenolic content, anthocyanin expression, and antimicrobial activity in the plant tissue, leading to a more robust, nutrient-dense crop.

- Red Light (600-700 nm): Red light aligns perfectly with the absorption peaks of chlorophyll a and b, making it highly efficient for driving raw photosynthesis and rapid canopy expansion. When combined with broad-spectrum white light, red light increases soluble sugar content and ascorbic acid (Vitamin C) synthesis22.

- UV-A/UV-B: Short-duration exposure to UV light (e.g., 10 minutes of UV-C prior to harvest) has been shown to temporarily stress the plant, inducing a defensive, rapid spike in chlorophyll and carotenoid content without permanently damaging the tissue.

To achieve optimal coloration and peak phytochemical density in demanding varieties like red cabbage or amaranth, a PPFD of 150 to 220  is generally required50. For standard microgreen biomass focused purely on yield, 120 to 180  is sufficient45.

This biological reality creates a complex, secondary environmental insight: there is a fundamental, inescapable trade-off between the energy intensity required to maximize specific phytonutrients (such as sulforaphane) and the resultant Global Warming Potential per kilogram of crop. Pushing a VivoSun or Mars Hydro fixture to deliver 250  of a blue-heavy spectrum will undoubtedly spike the glucosinolate profile, but it simultaneously increases the total kilowatt-hours consumed per harvest cycle. The Cotyledon software orchestrating this system must therefore dynamically balance the predictive nutritional yield against the environmental footprint, utilizing highly efficient diodes (>2.7 ) to decouple metabolic enhancement from fossil-fuel expenditure.

## Organic Hydroponic Nutrient Regimes and Eutrophication Potential

Because the system employs a purely liquid hydroponic matrix flowing over the hemp mats, the composition of the nutrient solution is the primary source of plant fertility. Standard commercial hydroponics rely on highly refined, highly soluble mineral salts (such as calcium nitrate and potassium phosphate) synthesized via the Haber-Bosch process, which is immensely energy-intensive and heavily reliant on natural gas as a hydrogen feedstock. To maintain strict organic integrity, the system utilizes natural biological inputs, specifically fish hydrolysate and liquid kelp.

### Fish Hydrolysate vs. Fish Emulsion

While both are marine-derived organic fertilizers, their respective manufacturing processes result in vastly different biological outputs and life cycle impacts. Fish emulsion is processed using intense heat to remove commercially valuable oils and separate proteins, resulting in a thick, odoriferous liquid. In stark contrast, fish hydrolysate is cold-processed, utilizing naturally occurring or introduced enzymes to break down whole fish or fish waste (bycatch, frames, offal) into a liquid form without the application of external thermal energy51.

The cold process of hydrolysate preserves the delicate, volatile oils, essential amino acids, and complex proteins that are routinely denatured and destroyed in the emulsion process51. From an LCA perspective, manufacturing fish hydrolysate avoids the severe thermal energy penalty associated with emulsions52. A comparative life cycle assessment of organic biostimulants indicates that fish hydrolysate represents a uniquely low-impact pathway for nitrogen and phosphorus delivery, especially when sourced from existing fishery waste streams, functioning as an upcycled, circular resource that mitigates the depletion of terrestrial mineral reserves52. Furthermore, liquid kelp (e.g., Ascophyllum nodosum) is integrated into the nutrient regime to supply naturally occurring phytohormones, notably cytokinins, which regulate cell division and significantly enhance stress tolerance in dense, highly competitive microgreen canopies54.

### Eutrophication Potential and Closed-Loop Systems

The primary environmental concern with applying high concentrations of nitrogen and phosphorus—regardless of whether they originate from organic marine sources or synthetic chemistry—is Eutrophication Potential (EP). Eutrophication occurs when excess nutrients run off into freshwater or marine ecosystems, triggering aggressive algal blooms. As these blooms die and decompose, they deplete dissolved oxygen in the water column, leading to hypoxic "dead zones" that devastate aquatic life9. EP is mathematically standardized using phosphate equivalents (); for example, 1 kilogram of ammonia emission represents 0.35 kg of , and 1 kilogram of Chemical Oxygen Demand (COD) emitted to freshwater equals 0.022 kg 10.

In traditional open-field agriculture, the application of organic fertilizers like fish hydrolysate often results in volatile ammonia emissions to the air and significant nitrate leaching into the groundwater table. However, in a hand-watered, closed-loop indoor rack system using precise 10x20 flats, aqueous runoff is completely captured, recirculated, or meticulously managed. This physical containment effectively reduces the aquatic freshwater and marine eutrophication potentials to near absolute zero during the operational phase38.

A unique biological challenge arises when substituting mineral hydroponic salts with organic fish hydrolysate in an otherwise sterile, soil-less system. The complex organic compounds in hydrolysate require a robust, active microbial population to mineralize the proteins into plant-bioavailable ammonium and nitrate. Without the deep buffering capacity of field soil, the liquid matrix can experience a rapid spike in Biological Oxygen Demand (BOD) as heterotrophic bacteria multiply to break down the fish oils46. This necessitates hyper-oxygenation of the nutrient reservoir—requiring electric air pumps and air stones—which introduces a minor but measurable addition to the electricity inventory within the LCA model.

## The Organic Seed Supply Chain and Upstream Footprints

In the conventional agricultural production of mature crops, the physical mass of the seed planted is statistically insignificant compared to the mass of the final harvested fruit or biomass. Microgreens present a stark inversion of this biological ratio. Harvested at the cotyledon stage, a standard 10x20 flat requires an incredibly dense sowing rate (e.g., 20 to 30 grams of seed) to yield roughly 200 to 300 grams of fresh biomass. Consequently, the seed itself constitutes a major fraction of the final product by weight, often representing 10% or more of the harvested mass.

LCA models of indoor microgreen production confirm this unique dynamic: seeds are one of the most dominant contributors to the total Global Warming Potential of the entire system. In rigorous assessments, the seed supply chain accounts for over 4.04  per kg of harvested microgreens, rivaling or even exceeding the carbon footprint of the electricity used to power the LED lights22.

### Tracing the Supply Chain

The operation procures its organic microgreen seeds—such as broccoli, sunflower, and fenugreek—from True Leaf Market, an independent e-commerce seed company headquartered in Salt Lake City, Utah58. While the distributor is centralized, the organic seeds are cultivated geographically worldwide, relying on an extensive global agricultural network.

The embedded carbon footprint of an organic seed encompasses the entire agricultural life cycle of the mother plant. To produce organic broccoli or sunflower seed, open-field agriculture requires:

- Tractor Operations and Diesel Consumption: Primary tillage, harrowing, and mechanical harvesting rely entirely on diesel fuel. In U.S. agriculture, diesel accounts for 44% of direct energy consumption59. An LCA calculating the energy input for seed production indicates that diesel and heavy machinery operations represent the highest non-fertilizer energy inputs, emitting roughly 56.31 MJ of energy per liter of diesel combusted59.

- Irrigation: Pumping groundwater for field crops requires significant electrical or diesel energy, deeply impacting the energy ratio (output vs. input energy) of the seed crop61.

- Organic Fertilization: While avoiding synthetic nitrogen (which drastically lowers the energy footprint compared to conventional farming), organic seed crops still require the transport, spreading, and incorporation of heavy composts and manures, which require machinery hours27.

- Processing and Transport: Once harvested, seeds must be dried, cleaned, tested for germination rates, packaged, and transported from global farms to the Utah distribution center, and subsequently shipped to the local CEA facility via freight64.

To accurately track this massive footprint within the Cotyledon software, standard baseline datasets are necessary. The USDA's Federal LCA Commons provides extensive digital unit process data regarding field crop production, greenhouse gas emissions from agriculture, and regional transportation logistics across the United States5. Integrating this upstream data ensures that the LCA model accounts for the hidden agricultural burdens of the organic seeds, preventing the illusion of a "zero-carbon" facility simply because the immediate indoor operation is clean16.

## The Comprehensive LCA Dataset Integration

To facilitate the software architecture of Cotyledon, the following dataset consolidates the critical parameters, emission factors, and energy efficacies across hardware, substrates, and biological inputs discussed throughout the research. This structured data serves as the foundation for real-time calculation matrices.

### Core LCA Parameters Database for Cotyledon Software

| Component / Input | Parameter / Metric | Value / Range | Unit | LCA Impact Category | Citations |
| --- | --- | --- | --- | --- | --- |
| Stainless Steel / Metal Racks | GWP (A1-A3 Manufacturing) | 2.78 - 3.62 |  | Global Warming Potential | 19 |
| Stainless Steel / Metal Racks | Reference Service Life | 50 | Years | Amortization / Durability | 19 |
| Peat Moss Substrate | GWP (Extraction) | High |  | GWP / Habitat Destruction | 25 |
| Coco Coir Pith | Methane emissions (Retting) | 0.0328 |  | GWP (Biogenic) | 7 |
| Hemp / Jute Mat | End of Life (Composting) | 100% | Biodegradable | Circularity / Waste Reduction | 33 |
| Barrina 4ft T8 LEDs | Power Draw (per 6-pack) | 180 - 252 | Watts | Energy Consumption | 40 |
| Mars Hydro / VivoSun LEDs | Photosynthetic Photon Efficacy | 2.7 - 2.96 |  | Energy Efficiency | 44 |
| Microgreen Lighting Req. | Target PPFD (Biomass) | 120 - 180 |  | Operational Energy | 50 |
| Microgreen Lighting Req. | Target PPFD (Phytonutrient) | 150 - 220 |  | Operational Energy | 45 |
| Organic Seeds (Field Grown) | GWP Contribution (Microgreens) | ~4.04 |  | GWP (Upstream) | 5 |
| Field Diesel Consumption | Energy Density | 56.31 |  | GWP / Energy Input | 61 |
| Fish Hydrolysate | Processing Energy | Low (Cold Process) |  | GWP Reduction vs Emulsion | 51 |
| Ammonia (Nutrient Runoff) | Eutrophication Equivalent | 0.35 |  | Eutrophication Potential | 10 |

## Dynamic LCA and Software Orchestration (Cotyledon Integration)

The traditional execution of a Life Cycle Assessment is a static, retrospective process resulting in a PDF report. The objective of the Cotyledon software is to transform LCA into a dynamic, predictive engine that actively manages business operations and farming production.

To achieve this, the software architecture leverages OpenLCA, a leading open-source life cycle modeling tool utilized by researchers and industry professionals globally67. OpenLCA offers a robust Inter-Process Communication (IPC) API, operating via JSON-RPC over HTTP or gRPC protocols, which allows external applications written in modern programming languages like Python, JavaScript, or C# to communicate directly with the LCA calculation kernel68.

By establishing a secure API connection between the Cotyledon app and an active OpenLCA server loaded with the Ecoinvent, Agribalyse, and USDA Federal LCA Commons databases3, the system automates continuous environmental auditing. The data flow functions through several critical stages:

- Inventory Data Acquisition: Sensors and manual inputs within the Cotyledon app record the exact weight of organic True Leaf Market seeds sown, the precise volume of fish hydrolysate dispensed, the hours of LED illumination at specific wattage parameters (differentiating between a Barrina T8 and a dimmed VivoSun AeroLight), and the total fresh weight of the harvested microgreens.

- API Execution: The application compiles this parameterized inventory data and sends it via JSON payload to the OpenLCA IPC server68.

- Real-Time LCIA Calculation: OpenLCA maps these operational inputs to the vast background database flows—for instance, calculating the footprint of 1 kWh of electricity based on the local grid's emission factor, amortizing the embodied carbon of the stainless steel rack over its 50-year life, and aggregating the upstream tractor diesel footprint of the organic seeds. It then calculates the exact Global Warming Potential and Eutrophication Potential for that specific harvest flat69.

- Optimization Feedback Loop: If the Cotyledon software detects that a specific variety of broccoli microgreen requires 200  of blue-heavy spectrum to achieve target sulforaphane levels22, it queries the LCA model to calculate the exact carbon penalty of that energy increase. The operator or the automated system can then adjust the lighting recipe, photoperiod, or switch to a higher-PPE fixture to balance nutritional output against the environmental bottom line.

This dynamic integration essentially constructs a digital twin of the farming operation. Through the seamless synthesis of physiological phytonutrient data, infrastructural embodied carbon matrices, and real-time operational inputs, the Cotyledon software renders the environmental cost of biological optimization transparent and actionable in real-time. The vertical farm is no longer operating blindly; it is actively resolving the complex calculus of sustainable agriculture, simultaneously maximizing human nutritional yield while verifiably minimizing the planetary ecological cost.

#### Works cited

- Broccoli Microgreens Sulforaphane Benefits Explained (2026), https://aquagertech.com/blogs/microgreens/sulforaphane-broccoli-microgreens

- Life Cycle Assessment (LCA) – Everything you need to know, https://ecochain.com/blog/life-cycle-assessment-lca-guide/

- Life Cycle Assessment (LCA) - Knowledge Base - Ecoinvent, https://support.ecoinvent.org/ecoinvent-use-cases

- On Life Cycle Assessment to Quantify the Environmental Impact of, https://www.led-professional.com/resources-1/articles/on-life-cycle-assessment-to-quantify-the-environmental-impact-of-lighting-products

- Life Cycle Assessment of a Prospective Technology for Building, https://www.mdpi.com/2073-4433/13/8/1317

- Environmental Life Cycle Assessment of GrowOff Modular Vertical, https://diva-portal.org/smash/get/diva2:1750912/FULLTEXT01.pdf

- Life Cycle Assessment of Various Filtering Media for Greywater, https://www.researchgate.net/publication/348746908_Life_Cycle_Assessment_of_Various_Filtering_Media_for_Greywater_Treatment_Using_Greenwall_Filtration_System

- Life-Cycle Assessment in the Polymeric Sector - PMC - NIH, https://pmc.ncbi.nlm.nih.gov/articles/PMC7361975/

- Eutrophication Potential - AgImpacts, https://agimpacts.mit.edu/indicators/eutrophication-potential/

- Eutrophication potential - Designing Buildings Wiki, https://www.designingbuildings.co.uk/wiki/Eutrophication_potential

- What is Ecoinvent? - Meet the LCI Database - Ecochain, https://ecochain.com/blog/what-is-ecoinvent/

- Life Cycle Assessment (LCA) – Conduct Studies with ecoinvent, https://ecoinvent.org/life-cycle-assessment/

- Link with Ecoinvent & WFLDB | AGRIBALYSE® documentation (EN), https://doc.agribalyse.fr/documentation-en/agribalyse-program/link-with-ecoinvent-and-wfldb

- Life Cycle Assessment Method | AGRIBALYSE® documentation (EN), https://doc.agribalyse.fr/documentation-en/data-use/life-cycle-assessment-method

- Agribalyse - openLCA Nexus: The source for LCA data sets, https://nexus.openlca.org/database/Agribalyse

- LCA Commons - Ag Data Commons - Figshare - USDA, https://agdatacommons.nal.usda.gov/articles/dataset/LCA_Commons/24660180

- Welcome to the Federal LCA Commons | Life Cycle Assessment, https://www.lcacommons.gov/

- Agri-footprint 5.0 - SimaPro, https://simapro.com/wp-content/uploads/2020/10/Agri-Footprint-5.0-Part-2-Description-of-data.pdf

- ENVIRONMENTAL PRODUCT DECLARATION - Ventilatieland, https://www.ventilatieland.nl/static/uploads/pictures/original/other/sk22277_Environmental_product_declaration_EN.pdf

- ENVIRONMENTAL PRODUCT DECLARATION - XL-BYGG, https://www.xlbygg.se/media/attachments/806/00d/80600da6bcc48f432c478f9e2767727b.pdf

- ENVIRONMENTAL PRODUCT DECLARATION - RSK Databasen, https://www.rskdatabasen.se/infodocs/EPD/EPD_1061_8824003.pdf

- Effects of LED lighting on the nutritional properties and microbial, https://www.frontiersin.org/journals/nutrition/articles/10.3389/fnut.2026.1869208/full

- Top 5 Best Growing Mediums for Microgreens - Bootstrap Farmer, https://www.bootstrapfarmer.com/blogs/microgreens/microgreens-growing-media

- Improving food security through indoor vertical farming of microgreens, https://www.frontiersin.org/journals/sustainable-food-systems/articles/10.3389/fsufs.2026.1809881/full

- Torf und Torfersatzprodukte im Vergleich - ZHAW, https://www.zhaw.ch/storage/lsfm/institute-zentren/iunr/oekobilanzierung/eymann-2015-lca-torf.pdf

- Comparative climate change impacts of different strawberry, https://aspace.agrif.bg.ac.rs/bitstream/handle/123456789/7746/bitstream_29123.pdf?sequence=1&isAllowed=y

- Recent advances in organic agriculture: innovations, challenges, https://pmc.ncbi.nlm.nih.gov/articles/PMC12827596/

- Best Growing Media for Microgreens: Soil, Coco Coir & Mats, https://mpseeds.eu/ultimate-guide-to-growing-media

- I. NATURAL FIBER-POLYOLEFIN COMPOSITES. MINI-REVIEW, https://cellulosechemtechnol.ro/pdf/CCT7-8(2014)/p.599-611.pdf

- Bacterial-Retted Hemp Fiber/PLA Composites - MDPI, https://www.mdpi.com/2227-9717/13/4/1000

- Coco Coir vs Hemp Grow Mats: An Honest Comparison | TerraFibre, https://terrafibre.ca/pages/coco-coir-vs-hemp-grow-mats

- Best Growing Mats for Microgreens: A Handy Comparison, https://rusticwise.com/best-growing-mats-for-microgreens/

- Manufacturing and Properties of Jute Fiber-Reinforced Polymer, https://pmc.ncbi.nlm.nih.gov/articles/PMC11901065/

- A Comparative Life Cycle Assessment of a Composite Component, https://www.aidic.it/cet/13/32/288.pdf

- Bio-based materials as a robust solution for building renovation, https://www.research-collection.ethz.ch/bitstreams/1151ecb3-4e28-4f23-b0b9-5fbc375f44b6/download

- Bio-based materials as a robust solution for building renovation, https://arodes.hes-so.ch/record/10059/files/Padey_2022_bio-based_materials_robust_solution_building_renovation.pdf

- Life Cycle Assessment of a modular LED luminaire and quantified, https://upcommons.upc.edu/bitstreams/7a8f3095-d7c1-4f9f-affb-7484099d6e76/download

- Consumer Preference for Microgreens in the Presence of LED, https://journals.ashs.org/view/journals/hortsci/58/3/article-p327.xml

- Life-Cycle Assessment of Energy and Environmental Impacts of LED, https://www.pnnl.gov/main/publications/external/technical_reports/pnnl-21443.pdf

- Barrina 4FT Plant Grow Lights 5000K Full Spectrum Daylight White, https://www.lowes.com/pd/Barrina-4FT-Plant-Grow-Lights-5000K-Full-Spectrum-Daylight-White-252W-LED-T8-Grow-Light-Strips-6-Pack/8013252

- Barrina LED Grow Lights Instruction Manual - Full Spectrum T8, 3ft, https://manuals.plus/asin/B0B76SJ5XF

- Barrina 4FT T8 Plant Grow Light Review, https://www.letsallgrowcannabis.com/picks/grow-lights/barrina-4ft-t8-plant-grow-light-review/

- What Wattage LED Grow Light Do You Need for a 4×4 Grow Tent?, https://vivosun.com/growing_guide/right-led-wattage-for-4x4-grow-tent/

- How Many Grow Lights Do You Need? - Vivosun, https://vivosun.com/growing_guide/how-many-grow-lights-do-i-need/

- VIVOSUN 4 Pack AeroLight 400W LED Grow Light Review, https://vivosun.com/growing_guide/vivosun-aerolight-grow-light-review/

- Grow Light Cost Calculator - Mars Hydro, https://www.mars-hydro.com/grow-light-cost-calculator

- Best LED Grow Lights 2026: Efficiency and PPE Compared - Trimleaf, https://trimleaf.com/blogs/guides/best-grow-lights

- Most Nutritious Microgreens: All 18 Varieties Ranked (2026), https://aquagertech.com/blogs/microgreens/microgreens-nutrition-comparison

- Effects of LED light treatments on the bioactive composition and, https://www.frontiersin.org/journals/plant-science/articles/10.3389/fpls.2026.1834435/full

- LED Light Recipe, PPFD and Spectrum Requirements for Microgreens, https://ceaunion.com/blog/vertical-farming/led-light-recipe-ppfd-and-spectrum-requirements-for-microgreens

- Fish Fertilizer - Is it Worth Buying? - Garden Myths, https://www.gardenmyths.com/fish-fertilizer-worth-buying/comment-page-3/

- Life cycle assessment of the LimoFish process for the ... - ChemRxiv, https://chemrxiv.org/doi/pdf/10.26434/chemrxiv-2022-6c3fd

- A Life Cycle and Environmental Cost Analysis in Denmark and, https://pubs.acs.org/doi/10.1021/acsagscitech.5c01013

- Liquid organic fertilizers in soilless cultivation: a systematic review of, https://www.frontiersin.org/journals/sustainability/articles/10.3389/frsus.2026.1775182/full

- Nutrients and Eutrophication | U.S. Geological Survey - USGS.gov, https://www.usgs.gov/mission-areas/water-resources/science/nutrients-and-eutrophication

- Sources and Solutions: Agriculture | US EPA, https://www.epa.gov/nutrientpollution/sources-and-solutions-agriculture

- Eutrophication Potential | Space4Water Portal, https://www.space4water.org/water/eutrophication-potential

- Celebrating Our Roots: Legacy of Our Garden Members, https://ngb.org/celebrating-members/

- A review on global energy use patterns in major crop production, https://pubs.rsc.org/va/article/1/5/662/794408/A-review-on-global-energy-use-patterns-in-major

- Energy consumption in agriculture increased in 2016, driven mainly, https://www.ers.usda.gov/data-products/charts-of-note/87964

- Energy use efficiency in paddy cultivation in Punjab, https://www.ecologyjournal.in/assets/archives/2021/vol3issue1/3-1-90-473.pdf

- Estimation of energy flow and environmental impacts of quinoa, https://pmc.ncbi.nlm.nih.gov/articles/PMC7245589/

- A Life Cycle Assessment of Organic and Chemical Fertilizers ... - MDPI, https://www.mdpi.com/2071-1050/14/7/3912

- Mastering Organic Seed Production for Better Crops - LoginEKO, https://www.logineko.com/knowledge/mastering-organic-seed-production-for-better-crops/

- A life cycle analysis (LCA) primer for the agricultural community, https://www.osti.gov/servlets/purl/1802622

- Global database of GHG emissions related to feed crops, https://openknowledge.fao.org/server/api/core/bitstreams/c3b15795-3030-41c4-986e-31a090aa2ab4/content

- openLCA.org | openLCA is a free, professional Life Cycle, https://www.openlca.org/

- Introduction - Inter-process communication with openLCA, https://greendelta.github.io/openLCA-ApiDoc/

- Features | openLCA.org, https://www.openlca.org/features/

- Concept | openLCA.org, https://www.openlca.org/concept/

- How to Calculate the PCF of My Vertical Farm in openLCA?, https://ask.openlca.org/8778/how-to-calculate-the-pcf-of-my-vertical-farm-in-openlca